Always show all work for full credit.

- Determine the largest (closest to +∞) & smallest (closest to -∞) values (in decimal) that can be represented with 7-bit number for each representation. Show the decimal interpretation of each.

a. Sign/magnitude

Largest:

Smallest:

b. Unsigned

Largest:

Smallest:

c. Two’s complement

Largest:

Smallest:

2. Fill in the missing values in the following table (assuming unsigned integer values):

| Binary | Decimal | Hex |
| --- | --- | --- |
|  | 42 |  |
|  | 221 |  |
|  |  | AC |
|  |  | 82 |

3. Convert the following numbers to 8-bit, two’s complement (show the binary representation).

a. 92

b. -101

4. Convert the following 8-bit, two’s complement numbers to decimal:

a. 0011 1001

b. 1011 0010

5. Perform the following additions of two’s complement numbers (show work, including carries):

a.  1001  0111				b. 0011  0101			c.   0111 1111

+ 0011 0100               			 + 0100 0011		 	  + 0000 0001

6. A majority gate produces a TRUE output if and only if more than half of its inputs are TRUE. Complete a truth table for the four-input majority gate shown (inputs are named A, B, C, and D.  Output is named Y).  Be sure to order the table using a binary-counting order, starting with A=0, B=0, C=0, D=0 in the first row, then A=0, B=0, C=0, D=1 in the second row, etc.

7. Convert the following 8-bit two’s complement numbers to 16-bit two’s complement numbers with the same value (show final answers in binary).

a. 0011 1001

b. 1001 1110